# ROV Dive Summary, EX-21-03, Dive 01

# June 15,2021

## General Location Map

Dive 01 named Currituck 1 on the shelf off of the North Carolina coast.

## Dive Information

| Site Name | Currituck 1 (Engineering) |
| --- | --- |
| General Area Descriptor | US Mid-Atlantic, Currituck Landslide |
| Science Team Leads | Karl McLetchie |
| Expedition Coordinator | Kasey Cantwell/Matt Dornback |
| ROV Dive Supervisor | Karl McLetchie |
| Mapping Lead | Shannon Hoy |
| Dive Purpose | The first engineering dive of the ROV Shakedown. Primary objectives include testing new motors, motor controllers, lights, cameras, and hydraulic systems on the ROVs. |
| Was the dive restricted for Underwater Cultural Heritage? | No |
| ROV Dive Summary Data | Dive Summary: EX2103_DIVE01 ^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^ Dive Type: Normal In Water: 2021-06-15T14:28:06.018530 36.42318896679038 ; -74.75313700799491 On Bottom: 2021-06-15T15:34:45.385618 36.413076641378815 ; -74.75137260689709 Off Bottom: 2021-06-15T18:52:06.889336 36.412851063291114 ; -74.75110113923999 Out Water: 2021-06-15T19:33:13.807068 36.40951123073365 ; -74.75003209315052 Dive Duration: 5:5:7 Bottom Time: 3:17:21 Max Vehicle Depth: 670.4 m Min Seafloor Depth: 665.2 m Distance Travelled: 15.9 m |
| Dive Description | This was an engineering dive on the Currituck landslide feature. Due to a lateral thruster issue on Deep Discoverer, the ROV made a soft landing on the seafloor and did not move anywhere else until ascent. The seafloor was a muddy silty texture. Crabs, worms, shrimp, and various fish were seen. |
| Notable Observations | Many decapod crabs, some feeding. |
| Community and habitat observations | Corals and Sponges - Absent Chemosynthetic Community - Absent High biodiversity Community - Absent Active Seep or Vent - Absent Extinct Seep or Vent - Absent Hydrates - Absent |
| CMECS Feature Type(s) | Flat, Submarine Slide Deposit |
| SeaTube Link (science annotation system) | https://data.oceannetworks.ca/SeaTubeV3?resourceTypeId=600&resourceId=2143 |

### Equipment Deployed

| ROV |  | Deep Discoverer |  |  |
| --- | --- | --- | --- | --- |
| Camera Platform |  | Seirios |  |  |
| ROV Measurements |  | The following ROV measurements, data streams and equipment are used on each ROV deployment: CTD, depth, scanning sonar, USBL position, altitude, heading, attitude, high-resolution cameras, low resolution cameras, manipulator arms, suction sampler, sample drawers and thrusters. The section below notes if any of these sensors were malfunctioning or not operational |  |  |
| Equipment Malfunctions |  | Turbidity Sensor, CTD (ROV) |  |  |

### Close-up Map of Main Dive Site

Hypack map of the Dive 01 waypoints. Depth is displayed by contour lines at 10 meter increments and by colors. Warm colors are shallower and cool colors are deeper.

### Representative Photos of the Dive

Various crabs and worms seen at the Dive 01 landing site.

A fish and shrimp seen during ascent of the ROV.

## Samples Collected -

No samples were collected

### Niskin Sampling Summary

No Niskin bottles were used

### Scientists Involved (provide name, email, affiliation)

| Name | Email | Affiliation |
| --- | --- | --- |
| Jason Chaytor | jchaytor@usgs.gov | USGS |

### Please direct inquiries to:

NOAA Office of Ocean Exploration & Research

1315 East-West Highway, SSMC3 RM 10210

Silver Spring, MD 20910

oceanexplorer@noaa.gov