# ROV Dive Summary, EX-21-03, Dive 06,  June 19, 2021

## General Location Map

Dive 06 named Caryn Seamount. This site is on Caryn Seamount outside of the U.S. Exclusive Economic Zone.

## Dive Information

| Site Name | Caryn Seamount |
| --- | --- |
| General Area Descriptor | An isolated seamount with volcanic origin rising out of the Sohm Abyssal Plain |
| Science Team Leads | Karl McLetchie |
| Expedition Coordinator | Kasey Cantwell/Matt Dornback |
| ROV Dive Supervisor | Karl McLetchie |
| Mapping Lead | Shannon Hoy |
| Dive Purpose | The sixth engineering dive of the ROV Shakedown. Primary objectives include pilot training, testing new motors, motor controllers, lights, cameras, and hydraulic systems on the ROVs. Secondary objectives include exploring Caryn Seamount and collecting characteristic geological and biological samples. |
| Was the dive restricted for Underwater Cultural Heritage? | No |
| ROV Dive Summary Data | Dive Summary: EX2103_DIVE06 ^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^ Dive Type: Normal In Water: 2021-06-19T12:24:46.076705 36.66883664795508 ; -67.92760693520448 On Bottom: 2021-06-19T14:38:05.459955 36.668942568193714 ; -67.92861380123425 Off Bottom: 2021-06-19T18:10:54.960449 36.669588201078014 ; -67.92902245622362 Out Water: 2021-06-19T20:21:43.534606 36.66115593094224 ; -67.94041954204027 Dive Duration: 7:56:57 Bottom Time: 3:32:49 Max Vehicle Depth: 3796.6 m Min Seafloor Depth: 3764.0 m Distance Travelled: 234.3 m |
| Dive Description | Successful first ROV dive on Caryn Seamount. We started on a ridge arm at 3797 m and traversed upslope. An objective of the dive was to introduce topography with increased complexity for pilot training, ROV systems testing, and calibration. The dive started in an area with rubble and soft sediment. Several dead sponge stalks with brittle stars on top in addition to other sponges were present. A few corals were seen throughout the dive as well as a diversity of sponges. Two sponges (Hexactinellid sp. and Geodia sp.) and a rock sample were collected. Dive was called ~ 15 minutes early due to increased sustained winds and seas. The clear water gave the video team a chance to dial in the ME-20 camera from Seirios to see the tether and surrounding terrain, try different D2 lighting settings for shots from the main HD, and demonstrate how well the new camera controls work. The ME-20 will be moved to D2 once in the shelter of Block Island on Tuesday morning. The rocky terrain allowed the pilots to verify functionality of the SeaKing scanning sonars on D2 and Seirios and to test out the Blueview multibeam sonar on D2. The Bueview worked very well, painting a picture of the seafloor out to 90m. Auto altitude & heading continued to work very well. Auto XY seemed to hold position well. Moves in Auto XY were done in all directions and the associated data was collected for analysis. The D2 DVL was adjusted to pick up the seafloor at 90m altitude. The Seirios altimeter worked well. The Seirios USBL stopped tracking below 3,000m. It is suspected that the head needs oil. A spare head was swapped on for the next dive. If it is the head then it may need to be sent to Linkquest. We would still have one spare for EX2104. |
| Notable Observations | Many corals and sponges present |
| Community and habitat observations | Corals and Sponges - Present Chemosynthetic Community - Absent High biodiversity Community - Absent Active Seep or Vent - Absent Extinct Seep or Vent - Absent Hydrates - Absent |
| CMECS Feature Type(s) | Seamount, Ledge, Slope |
| SeaTube Link (science annotation system) | https://data.oceannetworks.ca/SeaTubeV3?resourceTypeId=600&resourceId=2193 |

### Equipment Deployed

| ROV |  | Deep Discoverer |  |  |
| --- | --- | --- | --- | --- |
| Camera Platform |  | Seirios |  |  |
| ROV Measurements |  | The following ROV measurements, data streams and equipment are used on each ROV deployment: CTD, depth, scanning sonar, USBL position, altitude, heading, attitude, high-resolution cameras, low resolution cameras, manipulator arms, suction sampler, sample drawers and thrusters. The section below notes if any of these sensors were malfunctioning or not operational |  |  |
| Equipment Malfunctions |  | Turbidity sensor, Seirios USBL |  |  |

### Close-up Map of Main Dive Site

Hypack map of the Dive 06 waypoints. Depth is displayed by contour lines at 10 meter increments and by colors. Warm colors are shallower and cool colors are deeper.

### Representative Photos of the Dive

A coral with a worm associate

A rocky slope with sponges attached.

## Samples Collected -

| Sample ID | EX2103_D06_01B |
| --- | --- |
| Date (UTC) | 06/19/2021 |
| Time (UTC) | 15:46 |
| Depth (m) | 3791.227 |
| Latitude (decimal degrees) | 36.669140 |
| Longitude (decimal degrees) | -67.929 |
| Temp. (°C) | 2.451 |
| Field ID(s) | Hydrozoa |
| Comments |  |

| Sample ID | EX2103_D06_02G |
| --- | --- |
| Date (UTC) | 6/19/2021 |
| Time (UTC) | 1655 |
| Depth (m) | 3785.131 |
| Latitude (decimal degrees) | 36.66936 |
| Longitude (decimal degrees) | -67.9291 |
| Temp. (°C) | 2.53244 |
| Field ID(s) | Loose Rock - Mg Encrusted |
| Comments |  |

| Associates Sample ID | Field Identification | Count |
| --- | --- | --- |
| EX2103_D06_02G_A01 | Cnidaria Actinea | 1 |
| EX2103_D06_02G_A02 | Bryozoa (lace corals) | 1 |
| EX2103_D06_02G_A03 | Bryozoa (lace corals) | 1 |

| Sample ID | EX2103_D06_03B |
| --- | --- |
| Date (UTC) | 6/19/2021 |
| Time (UTC) | 1756 |
| Depth (m) | 3772.538 |
| Latitude (decimal degrees) | 36.66954 |
| Longitude (decimal degrees) | -679295 |
| Temp. (°C) | 2.18375 |
| Field ID(s) | Porifera Geodia |
| Comments |  |

### Niskin Sampling Summary

No Niskin bottles were used

### Scientists Involved (provide name, email, affiliation)

No scientists were involved in this dive

### Please direct inquiries to:

NOAA Office of Ocean Exploration & Research

1315 East-West Highway, SSMC3 RM 10210

Silver Spring, MD 20910

oceanexplorer@noaa.gov