# ROV Dive Summary, EX-21-03, Dive 10,  June 25, 2021

## General Location Map

Dive 10 named Uchupi Canyon. This site is in Uchupi Canyon off of New York and New Jersey.

## Dive Information

| Site Name | Uchupi Canyon |
| --- | --- |
| General Area Descriptor | Mid-Atlantic Canyons |
| Science Team Leads | Karl McLetchie |
| Expedition Coordinator | Kasey Cantwell/Matt Dornback |
| ROV Dive Supervisor | Karl McLetchie |
| Mapping Lead | Shannon Hoy |
| Dive Purpose | The tenth engineering dive of the ROV Shakedown. Primary objectives include pilot training, testing new motors, motor controllers, lights, cameras, and hydraulic systems on the ROVs. Secondary objectives include exploring the central area of Uchupi Canyon and collecting characteristic geological and biological samples. |
| Was the dive restricted for Underwater Cultural Heritage? | No |
| ROV Dive Summary Data | Dive Summary: EX2103_DIVE10 ^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^ Dive Type: Normal In Water: 2021-06-25T12:31:01.021160 39.544834571127616 ; -71.79154749684005 On Bottom: 2021-06-25T13:27:53.605485 39.542909425804574 ; -71.79292804193973 Off Bottom: 2021-06-25T20:09:28.044946 39.53934779246253 ; -71.80182858086877 Out Water: 2021-06-25T20:38:06.410574 39.53432208178928 ; -71.8024252627831 Dive Duration: 8:7:5 Bottom Time: 6:41:34 Max Vehicle Depth: 1286.3 m Min Seafloor Depth: 1047.4 m Distance Travelled: 714.3 m Mid Water Transects Summary ^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^ Number of Transects: 3 Transect 1 Start: 18:44:44 39.541000868020355 ; -71.79944667005088 End: 19:06:46 39.54072861623938 ; -71.79978170427357 Duration: 0:22:2 Depth: 500.0 m Transect 2 Start: 19:17:40 39.540289409426535 ; -71.79999472704898 End: 19:38:07 39.54030960599953 ; -71.80067574250059 Duration: 0:20:27 Depth: 450.0 m Transect 3 Start: 19:46:50 39.54042800137336 ; -71.80098799896996 End: 20:09:08 39.539371493922964 ; -71.80180700486164 Duration: 0:22:18 Depth: 375.0 m |
| Dive Description | Dive 10 was successfully completed on Uchupi Canyon. The ROV reached bottom at a depth of 1287m and proceeded to ascend the south canyon wall to 1020m. The slope was mostly soft bottom. Many fish were observed including synaphobranchid eels, blue hake, sol, scorpion fish, and a great interaction between a curious dogfish and D2. Urchins, hydroids, cup coral, anemones, crabs, shrimp, holothurians, and squid were seen. At 1400, the ROVs concluded the benthic portion of the dive and switched to “simulating” water column exploration. The ROVs did three 20 minute transects, one each at 500, 450, and 375. Numerous ctenophores were seen as well as chaetognath worms, lobate jellies, siphonophores, and laternfish. The low light camera was used to attempt to see bioluminescence. No bioluminescence was observed. |
| Notable Observations |  |
| Community and habitat observations | Corals and Sponges - Present Chemosynthetic Community - Absent High biodiversity Community - Absent Active Seep or Vent - Absent Extinct Seep or Vent - Absent Hydrates - Absent |
| CMECS Feature Type(s) | Submarine Canyon, Slope |
| SeaTube Link (science annotation system) | https://data.oceannetworks.ca/SeaTubeV3?resourceTypeId=600&resourceId=2233 |

### Equipment Deployed

| ROV |  | Deep Discoverer |  |  |
| --- | --- | --- | --- | --- |
| Camera Platform |  | Seirios |  |  |
| ROV Measurements |  | The following ROV measurements, data streams and equipment are used on each ROV deployment: CTD, depth, scanning sonar, USBL position, altitude, heading, attitude, high-resolution cameras, low resolution cameras, manipulator arms, suction sampler, sample drawers and thrusters. The section below notes if any of these sensors were malfunctioning or not operational |  |  |
| Equipment Malfunctions |  | Turbidity sensor |  |  |

### Close-up Map of Main Dive Site

Hypack map of the Dive 10 waypoints. Depth is displayed by contour lines at 10 meter increments and by colors. Warm colors are shallower and cool colors are deeper.

### Representative Photos of the Dive

A dogfish interacting with the ROV.

A solitary cup coral and a shrimp on a small boulder.

## Samples Collected -

No samples were collected

### Niskin Sampling Summary

No niskin samples were taken.

### Scientists Involved (provide name, email, affiliation)

| Name | Email | Affiliation |
| --- | --- | --- |
| Quinn Girasek | quinn.girasek@noaa.gov | NOAA Ocean Exploration |
| Allen Collins | allen.collins@noaa.gov | NOAA & NMNH |
| Nick Beaseo | nickbezio@gmail.com | University of Maryland |
| Rene Martin | rpmartin@ku.edu | Kansas University |
| Michael Vecchione | vecchiom@si.edu | NOAA & NMNH |
| Steve Haddok | haddock@mbari.org | MBARI |
| Dhugal Lindsay | dhugal@jamstec.go.jp | JAMSTEC |
| Kelsey Viator | ksviator2000@gmail.com | University of Louisiana at Lafayette |

### Please direct inquiries to:

NOAA Office of Ocean Exploration & Research

1315 East-West Highway, SSMC3 RM 10210

Silver Spring, MD 20910

oceanexplorer@noaa.gov